- Change router symbols, right click  symbol selection -> affinity-square-red  router (optional step)

- Turn on Auto-Idle PC for both routers

- Configure the following commands for router 1 and 2. The command enable password cisco is used on Cisco devices (like routers and switches) to set a password for privileged EXEC mode (also called enable mode). Only users who know the password can enter enable mode by typing the enable command from user EXEC mode.

- Capture packets on direct link using Wireshark before configuring ssh on R2

- Ip ? will tell the options available to be added with ip command. We’ll use domain-name to set up domain for ssh. Setting up a domain name for SSH on a Cisco device (such as a router or switch) is a required step for generating the RSA key pair used for secure SSH communication. This command requires a hostname and domain name to generate a fully qualified domain name (FQDN), which is used to create a unique key pair.

- CDP packets exchanged between routers in which they tell different information about each other like their model number, version number, 7. IP etc. Wireshark will capture this

- Generate rsa keys for encrypting packets. If take less than 760 bits as key size then will not support SSH version 2

- With transport input can set the protocol to use, we can even ste both telnet and ssh

- Set login password and then save configuration

- SSH into R2 from R1. Show run command shows all running configurations

We configured SSH in router 2. Will access router 2 from router 1

- See the TCP stream for any randomly chosen packet; we can see that the data is encrypted in SSH unlike in Telnet