Enhanced Interior Gateway Routing Protocol

- First worked on cisco routers

- Now works on any router

- Classless routing protocol

- A classless routing protocol is a routing protocol that includes subnet mask information (also called the prefix length) in its routing updates.

- Administrative distance is 90

- Metrics include bandwidth, load and reliability used

- RIP uses hop count but inefficient as does check bandwidth and maximum of 216 protocols can be used

- EIGRP and OSPF both check the bandwidth

- Default metric is bandwidth and delay

- Conversion rate is very fast

- Supports VLSM and Subnetting : VLSM allows you to divide an IP network into subnets of different sizes, based on actual need. This helps avoid wasting IP addresses and is a core concept in classless IP addressing.

- Uses DUAL algorithm

- Summarization can be done in any router

Configure the IP addresses of all inetrfaces on all 4 routers as usual using the following

enable

configure terminal

interface fastethernet 0/0

ip address 192.168.12.1 255.255.255.0

no sh

exit

do wr

Do the same for all other routers

router eigrp 1

network (network-address of directly connected networks)  repeat command for as many directly connected networks to router

no auto-summary